Notes From Hall D Tagger Magnet Design Review

- Double check 1006 permeability curves. May be inconsistent in TOSCA and ANSYS

- Need to conduct full error imposed resolution analysis

- TOSCA field profile needs integral fields

- Review 1% global field uniformity requirement. May be too loose.

- Committee recommended coming up with a calibration method for tagger rather than requiring tight specs. Use variable energies of electron beam

- May be advantageous to machine one pole flush with the yoke steel and let the other ride.

- Magnet steel should be vacuum annealed or  in argon. Otherwise, the permeability may change over time. (Ideal in Chicago may have vacuum anneal equip.

- Nickel plating on entire pole not necessary, but if cheap ok.

- Tonox in resin for coils should not be used.

- Use 316L (not 316LN Hard to find).

- Determine if NMR feedback is actually required.

- Revisit double pancake design for coils. The thermal issues are not a problem at low delta T.

- May be a good idea to preassemble vacuum chamber and poles in test lab prior to installation.

- If we reduce to ¼ o-ring, we may be able to remove compression bolts.

- After all holes are in steel, run back through ANSYS for a look at saturation effects.

- Committee believes our engineering support estimates are low by about 14mw.

- Need to add $100K for coils.

- Estimate for magnet may be $300K too low. ($1.25M needed for complete project)

- Need to show that the magnet geometry is optimized.

- Can tolerances be increased (if calibrating)

- Generally cost $1.50 per pound for 1006 magnet steel. $5/lb for completed magnet steel ($800K for 80T magnet)

- See Tommy and Paul for actual recent costs.