Things to discuss:

- Swap to 48-pin Eurocard connector

- Trace/space for DAC BGA

  - Current spacing rules

    - 4mil traces

    - 405um trace-to-ball

    - 405um trace-to-trace

    - 405um trace-to-pad

  - Recommended spacing

    - 3.937 mil (100um) traces

    - 405um trace-to-ball

    - 433um trace-to-trace

    - 433um trace-to-pad

- DACHEALTH net

  - Downscales SiPMBias31 to 0-5V ADC range

  - Is ~20MΩ of voltage divider going to be too noisy?

  - Shoot for 100uA at 20V=2mW

- DAC REF_IN

  - Current +3.3V reference sets full scale output to 0-165V

  - Do we need a +4V reference to set to 0-200V or is voltage divider OK?

- DAC channels 25-31 – TODO: route to backplane

- Silk-screen layout