SiPM Digital Control Board Specs

- 2500.514mil x 3790.509mil

- 6 layer

- 0.003” min trace, .0078” min space

- Permanent white epoxy silk screen

- SMT Pads

  - Top: 524

  - Bottom: 0

- PTH Drills	Size		Number

12.992mil	9

28mil		152

35.433mil	26

39.370mil	48

62.992mil	2

110.236mil 	2

- Non-plated Through Hole Drills

127.953mil	2

- Front-to-back registration less than 5mil

- Warp and twist 7mil per inch or better

- Layer stack-up

  - Silk-screen top		TSK

  - Solder mask top 	TMK

  - Top			TOP

  - Digital ground		DGND

  - +3.3V			PWR1

  - Analog ground		AGND

  - +1.2V/+5V		PWR2

  - Bottom			BOT

  - Solder mask bottom	BMK